GenRiver/NetLogo: How it works?

[A] Spatial scaling rule on water

- Downscaling water (flows/stocks/variables) from subcatchment level into patch level is done through multiplication of subcathment properties with relative area of each patch in the subcatchment (i.e. vegetation fraction).

- Upscaling water (flows/stocks/variables) from patch level into subcatchment level is simply done through summation.

- Upscaling water (flows/stocks/variables) from subcathment level into catchment level is done through summation, weighted by relative area of each subcatchment.

[B] Temporal scaling rule on water

The main output of the model is daily river flows from given observation point that accumulate:

- overland flows that reach the river on the same day of the process

- subsurface flows that reach the river the next day after the process

- ground water flows that reach the river for long period after the process

[C] Arrays

- Arrays are represented in a list or multiple lists.  For example: a parameter with 1-D array (N=5) is [1 2 3 4 5], a parameter with 2-D array (3x4): [[1 2 3 4] [5 6 7 8] [9 10 11 12]], and a parameter with 3-D array (3x2x1): [[[a] [b]][[c] [d]][[e] [f]]].

- NetLogo functions mostly used for lists or multiple lists calculation in the model are: map, sum and item.

[D] Procedures

The procedures in the model is as follows:

- Reading input data

- Checking input data

- Summarising rain time series

- Generating time series for warming up and for simulation

- Initialising before warming up

- Warming up

- Resetting after warming up

- Simulating

- Reporting

# [1] Reading input data

1.1. Description

The main input of the model is a scenario file, which specifically assembles data on driver variables. Physically, it is a text file, containing list of input data file names.

1.2. Parameters resulted by this procedure

- C_CattleSale [non-arrays]

- C_DailyIntake [non-arrays]

- C_DailyTramplingFrac [non-arrays]

- C_NutritionMortalityThreshold [non-arrays]

- D_FeedingIntoLakeSub? [1-D: subcatchment]

- D_GWFracToLakeSub [1-D: subcatchment]

- D_GWUtilizedPatch? [2-D: subcatchment, luc-type]

- D_GWUtilizationFractionPatch [1-D: subcatchment]

- D_InterceptDripRate [non-arrays]

- D_MaxInterceptDripDuration [non-arrays]

- daily-rain-data-set [3-D: subcatchment, year, day]

- daily-rain-input-file-name-set [1-D: subcatchment]

- daily-rain-year-set [2-D: subcatchment, year]

- dynamic-subcathment-properties-data-set [3-D: subcatchment, year, dynamic-subcathment-properties]

- dynamic-subcathment-properties-input-file-name-set [1-D: subcatchment]

- dynamic-subcathment-properties-year-set [2-D: subcatchment, year]

- G_GrassLitterConverter [non-arrays]

- G_GrassMortalityFrac [non-arrays]

- G_GrazingManureConverter [non-arrays]

- G_SurfaceLitterDecayFrac [non-arrays]

- G_SurfaceManurePatchDecayFrac [non-arrays]

- G_TramplingMultiplier [non-arrays]

- I_AreaSub [1-D: subcatchment]

- I_DamHereSub? [1-D: subcatchment]

- I_GrassFracBiomassLUC [1-D: luc type]

- I_InitRelGW [non-arrays]

- I_InitRelSoil [non-arrays]

- I_InterceptionEffectOnTranspiration [non-arrays]

- I_InterceptLUC [1-D: luc type]

- I_LakeTranspirationMultiplier [non-arrays]

- I_MaxInfiltration [non-arrays]

- I_MaxInfiltrationSubSoil [non-arrays]

- I_ObserveAllSubcatchmentsObs? [2-D: subcatchment, observation point]

- I_PercolationFraction [non-arrays]

- I_Rain_GenSeed [non-arrays]

- I_Rain_IntensityCV [non-arrays]

- I_Rain_IntensityMax [non-arrays]

- I_Rain_IntensityMean [non-arrays]

- I_Rain_IntensityMin [non-arrays]

- I_RainMultiplier [non-arrays]

- I_RelDroughtLUC [1-D: luc type]

- I_ReservoirDepth [non-arrays]

- I_RiverFlowDispersalFactor [non-arrays]

- I_RoutingDistanceObs [2-D: subcatchment, observation point]

- I_RoutingVelocity_m_per_s [non-arrays]

- I_Tortuosity [non-arrays]

- L_FloodTresh [non-arrays]

- L_LakeBottomElev [non-arrays]

- L_LakeElevPostHEPP [non-arrays]

- L_LakeElevPreHEPP [non-arrays]

- L_LakeLevelFullHEPP [non-arrays]

- L_LakeLevelHalfHEPP [non-arrays]

- L_LakeLevelNoHEPP [non-arrays]

- L_LakeOverFlow [non-arrays]

- L_LakeOverFlowFrac [non-arrays]

- L_m3_per_kWh [non-arrays]

- L_QHEPP_m3_per_sec [non-arrays]

- L_QSanitaryFlow_m3_per_sec [non-arrays]

- lake-parameters-data-set [1-D: lake-parameters]

- lake-parameters-input-file-name [non-arrays]

- luc-fraction-data-set [3D: subcatchment, year, luc type]

- luc-fraction-input-file-name-set [1-D: subcatchment]

- luc-fraction-year-set [2D: subcatchment, year]

- monthly-evapotranspiration-data-set [non-arrays]

- monthly-evapotranspiration-input-file-name [1-D: month]

- monthly-relative-evapotranspiration-data-set [2-D: month, luc type]

- monthly-relative-evapotranspiration-input-file-name [non-arrays]

- observation-point-number [non-arrays]

- observed-river-flow-data-set [2-D: year, day]

- observed-river-flow-input-file-name [non-arrays]

- observed-river-flow-year-set [1-D: day]

- rain-parameters-data-set [1-D: rain-parameters]

- rain-parameters-input-file-name [non-arrays]

- reservoir-use-fraction-data-set [3-D: subcatchment, year, day]

- reservoir-use-fraction-input-file-name-set [1-D: subcatchment]

- reservoir-use-fraction-year-set [2-D: subcatchment, year]

- river-parameters-data-set [1-D: river-parameters]

- river-parameters-input-file-name [non-arrays]

- routing-distance-input-file-name[1-D: subcatchment]

- S_MaxTrampling [non-arrays]

- scenario-file-name [non-arrays]

- soil-parameters-data-set [1-D: soil-parameters-data-set]

- soil-parameters-input-file-name [non-arrays]

- static-luc-properties-data-set [1-D: static-luc-properties]

- static-luc-properties-input-file-name [non-arrays]

- static-subcathment-properties-data-set [1-D: static-subcathment-properties]

- static-subcathment-properties-input-file-name [non-arrays]

1.3. Related procedures in the script

- define-cattle-parameters-input-file-name

- define-daily-rain-input-file-name

- define-dynamic-subcathment-properties-input-file-name

- define-lake-parameters-input-file-name

- define-luc-fraction-input-file-name

- define-monthly-evapotranspiration-input-file-name

- define-monthly-relative-evapotranspiration-input-file-name

- define-observed-river-flow-input-file-name

- define-rain-parameters-input-file-name

- define-reservoir-use-fraction-input-file-name

- define-river-parameters-input-file-name

- define-routing-distance-input-file-name

- define-soil-parameters-input-file-name

- define-static-luc-properties-input-file-name

- define-static-subcathment-properties-input-file-name

- initialize-before-warming-up

- manage-input-file

- open-scenario

- read-cattle-parameters

- read-col-wise-text-file

- read-daily-rain

- read-dynamic-subcatchment-properties

- read-input-parameters

- read-lake-parameters

- read-luc-fraction

- read-monthly-evapotranspiration

- read-monthly-relative-evapotranspiration

- read-observed-river-flow

- read-rain-parameters

- read-reservoir-use-fraction

- read-river-parameters

- read-routing-distance

- read-row-wise-text-file

- read-soil-parameters

- read-static-luc-properties

- read-static-subcatchment-properties

- read-text-file

- read-time-series-data

- setup

# [2] Checking input data

2.1. Description

Since the input data come from various files, this procedure is aimed to check that all input data in Procedure 1 meet the model requirements, before proceeding to the next procedures. The check includes the check on number of parameters and consistency on number of dimensions.

2.2. Parameters resulted by this procedure:

- luc-number [non-arrays]

- subcatchment-number [non-arrays]

2.3. Related procedures in the script

- check-cattle-parameters

- check-days-number

- check-dynamic-subcathment-properties

- check-input-data-error

- check-lake-parameters

- check-luc-number

- check-rain-parameters

- check-river-parameters

- check-soil-parameters

- check-static-luc-properties

- check-static-subcathment-properties

- check-subcatchment-number

- initialize-before-warming-up

- setup

# [3] Summarising rain time series

3.1. Description

Input rain time series read by Procedure 1 may come from observed or simulated data (e.g. from SpatRain).  All type of data source should include information on the year, in which the data set represent. If the input time series do not cover certain year in the simulation, data for that year will be randomised using normal distribution based on monthly statistic.  Thus, this procedure is aimed to summarise statistic of monthly rainfall from each subcatchment, based on the input time series.

3.2. Parameters resulted by this procedure:

- max-daily-rain-data-set [2-D: subcatchment, month]

- mean-monthly-rain-data-set [2-D: subcatchment, month]

- min-daily-rain-data-set [2-D: subcatchment, month]

- p-monthly-wet-days-data-set [2-D: subcatchment, month]

- sd-monthly-rain-data-set [2-D: subcatchment, month]

3.3. Related procedures in the script

- read-input-parameters

- setup

- summarise-rain-data

# [4] Generating time series for warming up and for simulation

See also: Procedure 5, Procedure 6, Procedure 7

4.1. Description

Initial values of water in soil water, ground water and reservoir water stocks are initialised through a warming-up procedure, in order to make them independent with user initialisation. This procedure is called warming up.  In its simulation, the model requires some information in time series: daily rainfall, daily (relative) evapotranspiration, daily reservoir use fraction, yearly dynamic soil properties and yearly land use/cover fraction.  Thus, this procedure is aimed to generate those time series, which are separated for warming up purpose and for simulation purpose.  For daily rainfall time series, the data generation is simply done by reading the input time series for the simulation years that are inside the input time series or by randomisation using normal distribution for the simulation years that are beyond the input time series. For daily (relative) evapotranspiration time series, the data generation is simply done by repeating monthly data into daily data. For other time series, the data generation is simply done by reading the input time series for the simulation years that are inside the input time series or by inter/extrapolating the input time series for the simulation years that are beyond the input time series.

4.2. Parameters resulted by this procedure

- D_ReservoirUseFracSub_Simulation [3D: subcatchment, year, day]

- D_ReservoirUseFracSub_WarmUp [3D: subcatchment, year, day]

- generated-dynamic-subcathment-properties-data-set-for-simulation [3D: subcatchment, year, dynamic-subcathment-properties]

- generated-dynamic-subcathment-properties-data-set-for-warm-up [3D: subcatchment, year, dynamic-subcathment-properties]

- I_AvailWaterSub_Simulation [2D: subcatchment, year]

- I_AvailWaterSub_WarmUp [2D: subcatchment, year]

- I_BD_BDRefSub_Simulation [2D: subcatchment, year]

- I_BD_BDRefSub_WarmUp [2D: subcatchment, year]

- I_DailyRainSub_Simulation [3D: subcatchment, year, day]

- I_DailyRainSub_WarmUp [3D: subcatchment, year, day]

- I_ET_DailySet[non-arrays]

- I_GWRelFracSub_Simulation [2D: subcatchment, year]

- I_GWRelFracSub_WarmUp [2D: subcatchment, year]

- I_MaxDynGWSub_Simulation [2D: subcatchment, year]

- I_MaxDynGWSub_WarmUp [2D: subcatchment, year]

- I_PWPSub_Simulation [2D: subcatchment, year]

- I_PWPSub_WarmUp [2D: subcatchment, year]

- I_RelAreaPatch_Simulation [3D: subcatchment, year, luc type]

- I_RelAreaPatch_WarmUp [3D: subcatchment, year, luc type]

- I_RelETLUC_DailySet [2D: luc type, day]

- I_RiverFlowTimeSub_Simulation  [2D: subcatchment, year]

- I_RiverFlowTimeSub_WarmUp [2D: subcatchment, year]

- I_SoilFieldCapacitySub_Simulation [2D: subcatchment, year]

- I_SoilFieldCapacitySub_WarmUp [2D: subcatchment, year]

4.3. Related procedures in the script

- extract-2d-data-set

- generate-daily-evapotranspiration

- generate-daily-rain

- generate-dynamic-subcathment-properties

- generate-luc-fraction

- generate-reservoir-use-fraction

- inter-or-extrapolate-time-series-data

- randomize-time-series-data

- read-input-parameters

- setup

# [5] Initialising before warming up

See also: Procedure 4, Procedure 6, Procedure 7

5.1. Description

Initial values of water in soil water, ground water and reservoir water stocks are initialised through a warming-up procedure, in order to make them independent with user initialisation.  This procedure is called warming up. This procedure is aimed to initialise some parameters before warming up.

5.2. Parameters initialised by this procedure and their initial values:

- adjusted-observed-river-flow-year-set = observed-river-flow-year-set – simulation-start-year [1-D: year]

- C_Stocking = 1 [non-arrays]

- D_AtmWaterPatch = 0 [2-D: subcatchment, luc type]

- D_CumRiverFlowObs = 0 [2-D: subcatchment, observation point]

- D_DelaySurfaceFlowToRiverObs_DailySet [3D: subcatchment, year, day]

- D_DelaySurfaceFlowToRiverObs_DailySet = 0 [3-D: subcatchment, observation point, day]

- D_GWSub = I_MaxDynGWSub_Simulation[*,0] * I_InitRelGW [1D: subcatchment]

- D_MaxReservoirVolSub = if L_LakeHereSub? then I_AreaSub * I_ReservoirDepth else 0 [1-D: subcatchment]

- D_ReservoirVolSub = if I_DamHereSub? then D_MaxReservoirVolSub else 0 [1-D: subcatchment]

- D_SoilWaterPatch = (I_AvailWaterSub_Simulation[*,0] * I_InitRelSoil * *I_RelAreaPatch) [2D:subcatchment, luc type]

- D_TotalDelayRiverFlowObs = 0 [2-D: subcatchment, observation point]

- G_GrassBiomassPatch = 1 [2-D: subcatchment, luc type]

- G_SurfaceLitterPatch = 0 [2-D: subcatchment, luc type]

- G_SurfaceManurePatch = 0 [2-D: subcatchment, luc type]

- L_CumEvapLake = 0 [non-arrays]

- L_CumHEPPUse = 0 [non-arrays]

- L_CumRiverOutflow = 0 [non-arrays]

- L_HEPPDailyDemand = if (sum(I_AreaSub[*] > 0) then (L_QHEPP_m3_per_sec * 3600 * 24) / (sum(I_AreaSub[*])*10^-3) else 0 [non-arrays]

- L_LakeAreaSub if L_LakeHereSub? then I_AreaSub else 0 [1-D: subcatchment]

- L_LakeHereSub? = I_RelAreaPatch[*,j] > 0, where j is water body [1-D: subcatchment]

- L_LakeVol = if L_HEPP_Active? then L_OutflowTresholdVolPostHEPP else L_OutflowTresholdVolPreHEPP [non-arrays]

- L_OutflowTresholdVolPostHEPP = 1000 * (L_LakeElevPostHEPP – L_LakeBottomElev) * sum(L_LakeAreaSub[*]) [non-arrays]

- L_OutflowTresholdVolPreHEPP = 1000 * (L_LakeElevPreHEPP-L_LakeBottomElev) * sum(L_LakeAreaSub[*]) [non-arrays]

- L_SanitaryFlow = if(sum I_AreaSub[*] > 0)  then (L_QSanitaryFlow_m3_per_sec * 3600 * 24) / (sum(I_AreaSub[*])*10^-3) else 0 [non-arrays]

- O_CumDebitCatch = 0

- O_CumDeepInfiltrationCatch = 0 [non-arrays]

- O_CumETCatch = 0 [non-arrays]

- O_CumGWCatch = sum(I_MaxDynGWSub_Simulation[*,0] * I_RelAreaSub) * I_InitRelGW [non-arrays]

- O_CumGWDischargeCatch = 0 [non-arrays]

- O_CumHEPP_kWh = 0 [non-arrays]

- O_CumInfiltrationCatch = 0 [non-arrays]

- O_CumInterceptCatch = 0 [non-arrays]

- O_CumPercolationCatch = 0 [non-arrays]

- O_CumRainCatch = 0 [non-arrays]

- O_CumSimulatedDebit = 0 [1-D: observation point]

- O_CumSoilDischargeCatch = 0 [non-arrays]

- O_CumSoilWaterCatch = sum(I_AvailWaterSub_Simulation[*,0] * I_RelAreaSub) * I_InitRelSoil [non-arrays]

- O_CumSurfaceFlowCatch = 0 [non-arrays]

- S_RelBDPatch = I_BD_BDRefSub_Simulation[*,0] [2-D: subcatchment, luc type]

- S_RippingSurfacePatch = 0 [2-D: subcatchment, luc type]

5.3. Related procedures in the script

- initialize-1D-parameters

- initialize-2D-parameters

- initialize-before-warming-up

- initialize-cattle

- initialize-grass

- initialize-lake

- initialize-output

- initialize-patch-water-balance

- initialize-soil-structure-dynamics

- initialize-stream-network

- initialize-subcathment

- setup

# [6] Warming up

See also: Procedure 4, Procedure 5, Procedure 7, Procedure 8

6.1. Description

This procedure is aimed for warming up purposes, since Initial values of water in soil water, ground water and reservoir water stocks are initialised through a warming-up procedure, in order to make them independent with user initialisation. Thus, this procedure is exactly simulating the model (as described in Procedure 8), but by using some input parameters generated specifically for warming up purposes, as described in Procedure 4.

6.2. Specific input parameters used by this procedure:

- D_ReservoirUseFracSub_WarmUp [3-D: subcatchment, year, day]

- I_AvailWaterSub_WarmUp [2-D: subcatchment, year]

- I_BD_BDRefSub_WarmUp [2-D: subcatchment, year]

- I_DailyRainSub_WarmUp [3-D: subcatchment, year, day]

- I_GWRelFracSub_WarmUp [2-D: subcatchment, year]

- I_MaxDynGWSub_WarmUp [2-D: subcatchment, year]

- I_PWPSub_WarmUp [2-D: subcatchment, year]

- I_RelAreaPatch_WarmUp [3-D: subcatchment, year, luc type]

- I_RiverFlowTimeSub_WarmUp [2-D: subcatchment, year]

- I_SoilFieldCapacitySub_WarmUp [2-D: subcatchment, year]

- warm-up-duration [non-arrays]

6.3. Parameters calculated by this procedure:

See Procedure 8.

6.4. Related procedures in the script

- initialize-before-simulation

- parameterize-daily-parameters

- parameterize-yearly-parameters

- report-and-plot

- setup

- simulate

- simulate-daily-dynamics

- simulate-grass-dynamics

- simulate-lake-dynamics

- simulate-patch-dynamics

- simulate-river-dynamics

- simulate-soil-structure-dynamics

- simulate-yearly-dynamics

- update-cattle-stocks

- update-data-set

- update-grass-stocks

- update-lake-stocks

- update-output-stocks

- update-patch-stocks

- update-river-stocks

- update-soil-structure-stocks

- update-stocks

- update-subcatchment-stocks

- warm-up

# [7] Resetting after warming up

See also: Procedure 4, Procedure 5, Procedure 6

7.1. Description

After warming up procedure, as described in Procedure 6, some parameters are reset.  This procedure is aimed to reinitialise some parameters after warming up before running the main simulation.

7.2. Parameters reset by this procedure and their reset values:

- C_Stocking = 1 [non-arrays]

- D_AtmWaterPatch = 0 [2-D: subcatchment, luc type]

- O_CumDebitCatch = 0 [2-D: subcatchment, luc type]

- D_CumRiverFlowObs = 0 [2-D: subcatchment, observation point]

- G_GrassBiomassPatch = 1 [2-D: subcatchment, luc type]

- G_SurfaceLitterPatch = 0 [2-D: subcatchment, luc type]

- G_SurfaceManurePatch = 0 [2-D: subcatchment, luc type]

- L_CumEvapLake = 0 [non-arrays]

- L_CumHEPPUse = 0 [non-arrays]

- L_CumRiverOutflow = 0 [non-arrays]

- O_CumDeepInfiltrationCatch = 0 [non-arrays]

- O_CumETCatch = 0 [non-arrays]

- O_CumGWCatch = sum(I_MaxDynGWSub_Simulation[*,0] * I_RelAreaSub) * I_InitRelGW [non-arrays]

- O_CumGWDischargeCatch = 0 [non-arrays]

- O_CumHEPP_kWh = 0 [non-arrays]

- O_CumInfiltrationCatch = 0 [non-arrays]

- O_CumInterceptCatch = 0 [non-arrays]

- O_CumPercolationCatch = 0 [non-arrays]

- O_CumRainCatch = 0 [non-arrays]

- O_CumSimulatedDebit = 0 [1-D: observation point]

- O_CumSoilDischargeCatch = 0 [non-arrays]

- O_CumSoilWaterCatch = sum(I_AvailWaterSub_Simulation[*,0] * I_RelAreaSub) * I_InitRelSoil [non-arrays]

- O_CumSurfaceFlowCatch = 0 [non-arrays]

- S_RelBDPatch = I_BD_BDRefSub_Simulation[*,0] [2-D: subcatchment, luc type]

7.3. Related procedures in the script

- reset

- setup

# [8] Simulating

See also: Procedure 6

8.1. Description

This is the main procedure of the model that calculates all dynamics.  This procedure is also used by warming up procedure (Procedure 6). Using this procedure, the main simulation differs with warming up simulation in its specific input parameters, as generated by Procedure 4.

8.2. Specific input parameters used by this procedure:

- D_ReservoirUseFracSub_Simulation [3-D: subcatchment, year, day]

- I_AvailWaterSub_Simulation [2-D: subcatchment, year]

- I_BD_BDRefSub_Simulation [2-D: subcatchment, year]

- I_DailyRainSub_Simulation [3-D: subcatchment, year, day]

- I_GWRelFracSub_Simulation [2-D: subcatchment, year]

- I_MaxDynGWSub_Simulation [2-D: subcatchment, year]

- I_PWPSub_Simulation [2-D: subcatchment, year]

- I_RelAreaPatch_Simulation [3-D: subcatchment, year, luc type]

- I_RiverFlowTimeSub_Simulation [2-D: subcatchment, year]

- I_SoilFieldCapacitySub_Simulation [2-D: subcatchment, year]

- simulation-duration [non-arrays]

8.3. Parameters simulated by this procedure and their equations

8.3.1. Parameters initialised before calculating the dynamic part

- I_Simulation_Time = 0 [non-arrays]

8.3.2. Parameters calculated before calculating the dynamic part

- I_RelAreaSub = I_AreaSub/sum(I_AreaSub) [1-D: subcatchment]

8.3.3. Parameters read from generated time series in yearly step

- I_AvailWaterSub  [1-D: subcatchment]

- I_BD_BDRefSub [1-D: subcatchment]

- I_GWRelFracSub [1-D: subcatchment]

- I_MaxDynGWSub [1-D: subcatchment]

- I_PWPSub [1-D: subcatchment]

- I_RelAreaPatch [2-D: subcatchment, luc type]

- I_RiverFlowTimeSub [1-D: subcatchment]

- I_SoilFieldCapacitySub [1-D: subcatchment]

8.3.4. Parameters read from generated time series in daily step

- D_ReservoirUseFracSub [1-D: subcatchment]

- I_DailyRainSub = I_DailyRainSub_WarmUp * I_RainMultiplier or I_DailyRainSub = I_DailyRainSub_Simulation * I_RainMultiplier [1-D: subcatchment]

- I_ET = I_ET_DailySet [non-arrays]

- I_MeasuredDebit_m3_per_sec [non-arrays]

- I_RelETLUC [1-D: luc type]

8.3.5. Non stocks-parameters calculated in the dynamic part

- C_DeathRate = C_Stocking * (min(1,(if(C_NutritionMortalityThreshold>0) then (C_RelNutrition / C_NutritionMortalityThreshold) else 0))) [non-arrays]

- C_Destocking = min(C_Stocking+C_StockingRate,C_CattleSale + C_DeathRate) [non-arrays]

- C_RelNutrition = if (C_Stocking * C_DailyIntake>0) then (min(1,sum(G_GrazingPatch[*,*])/(C_Stocking * C_DailyIntake)) else 1 [non-arrays]

- C_StockingRate = 0 * C_Stocking [non-arrays]

- C_TramplingCompactionPatch = if (C_DailyIntake>0) then (C_DailyTramplingFrac * G_GrazingPatch / C_DailyIntake) else 0 [2-D: subcatchment, luc type]

- D_ActETPatch = if (L_LakeHereSub?) then min(D_SoilWaterPatch+D_InfiltrationPatch,max(0,(((I_PETPatch – I_InterceptionEffectOnTranspiration * D_InterceptPatch)) * D_RelAvailWaterPatch))) else 0 [2-D: subcatchment, luc type]

- D_DeepInfiltrationSub = if (L_LakeHereSub?) then 0 else max(0,min(sum(I_DailyRainPatch[i,*])-sum(D_InterceptPatch[i,*])-sum(D_InfiltrationPatch[i,*]), I_MaxDynGWSub-D_GWSub, (sum(I_MaxInfiltrationPatch[i,*])* I_InfiltrationDurationSub/24) – (sum(I_SoilSaturationPatch[i,*])+sum(D_SoilWaterPatch[i,*])), I_MaxInfiltrationSubSoilPatch)) [1-D: subcatchment]

- D_DelayRiverFlowObs =  min(D_TotalDelayRiverFlowObs+D_DelaySurfaceFlowToRiverObs+D_DirectSurfaceFlowObs,if (not D_FeedingIntoLakeSub?) then (I_RelFracObs*D_TotalDelayRiverFlowObs) else 0) [2-D: subcatchment, observation point]

- D_DelaySurfaceFlowObs =  if (D_RoutingTimeObs>=1) then D_TotalStreamInflowSub else 0 [2-D: subcatchment, observation point]

- D_DelaySurfaceFlowToRiverObs =  D_DelaySurfaceFlowToRiverObs_DailySet[t], where t is simulation time [2-D: subcatchment, observation point]

- D_DirectRiverFlowObs = if(D_RoutingTimeObs >= 0 and D_RoutingTimeObs < 1) then (if(not D_FeedingIntoLakeSub?) then (D_TotalStreamInflowSub*I_RelFracObs) else 0) else 0  [2-D: subcatchment, observation point]

- D_DirectSurfaceFlowObs =  if (D_RoutingTimeObs>=0 and D_RoutingTimeObs<1) then (D_TotalStreamInflowSub*(1-I_RelFracObs)) else 0 [2-D: subcatchment, observation point]

- D_GWDischargeSub = min(D_GWSub+D_DeepInfiltrationSub+D_PercolationPatch,(D_GWSub * I_GWRelFracSub [1-D: subcatchment]))

- D_GWLakeObs = I_RelFracObs*D_GWFracToLakeSub*D_GWDischargeSub [2-D: subcatchment, observation point]

- D_GWToLakeObs = sum(D_GWLakeObs*I_RelAreaSub[*]) [1-D: observation point]

- D_InfiltrationPatch = if (L_LakeHereSub?)  then 0 else (max(0,min(I_SoilSaturationPatch- D_SoilWaterPatch, I_MaxInfiltrationPatch*I_InfiltrationDurationSub/24, I_DailyRainPatch-D_InterceptPatch))) [2-D: subcatchment, luc type]

- D_InterceptDurationSub = min(D_MaxInterceptDripDuration, sum(D_InterceptPatch[i,*]) /D_InterceptDripRate) [1-D: subcatchment]

- D_InterceptPatch = if (I_InterceptCapacityPatch > 0) then (I_InterceptCapacityPatch * (1 – exp (- I_DailyRainPatch / I_InterceptCapacityPatch))) else 0 [2-D: subcatchment, luc type]

- D_PercolationPatch = if (I_AvailWaterPatch> 0) then min(max(0,D_SoilWaterPatch+D_InfiltrationPatch- D_ActETPatch),max(0,(min(I_MaxDynGWSub-D_GWSub, I_MaxInfiltrationSubSoilPatch,D_SoilWaterPatch*I_PercolationFraction*I_GWRelFracSub))))-(if D_GWUtilizedPatch? then D_GWSub * D_GWUtilizationFractionPatch * (1-D_RelAvailWaterPatch) else 0) else -(if D_GWUtilizedPatch? then D_GWSub * D_GWUtilizationFractionPatch * (1-D_RelAvailWaterPatch) else 0) [2-D: subcatchment, luc type]

- D_RelAvailWaterPatch = if (I_AvailWaterClass* I_RelDroughtLUC > 0) then D_SoilWaterPatch / (I_AvailWaterClass* I_RelDroughtLUC) else 1 [2-D: subcatchment, luc type]

- D_ReservoirEvapSub = if(L_LakeHereSub?) then min(D_ReservoirVolSub+D_ReservoirInflowSub,I_ET_DailySet)  else 0 [1-D: subcatchment]

- D_ReservoirInflowSub = if(I_DamHereSub?)  then (D_GWDischargeSub+sum(D_SoilDischargePatch[i,*])+ D_SurfaceFlowSub) else 0 [1-D: subcatchment]

- D_ReservoirOutflowSub = min(max(0,D_ReservoirVolSub+D_ReservoirInflowSub-D_ReservoirEvapSub),if (L_LakeHereSub?) (if (D_ReservoirVolSub / D_MaxReservoirVolSub > 1) then (D_ReservoirVolSub – D_MaxReservoirVolSub) else (D_ReservoirUseFracSub*D_ReservoirVolSub))) else 0 [1-D: subcatchment]

- D_RiverFlowToLakeObs = sum(D_RiverToLakeSameDayObs*I_RelAreaSub)) + sum(D_RiverInflowToLakeObs*I_RelAreaSub) [1-D: observation point]

- D_RiverInflowToLakeObs =  min(max(0,D_TotalDelayRiverFlowObs+D_DelaySurfaceFlowToRiverObs+D_DirectSurfaceFlowObs-D_DelayRiverFlowObs),if (D_FeedingIntoLakeSub?) then (I_RelFracObs*D_TotalDelayRiverFlowObs) else 0)[2-D: subcatchment, observation point]

- D_RiverToLakeSameDayObs =  if (D_RoutingTimeObs>=0 and D_RoutingTimeObs < 1) then (if(D_FeedingIntoLakeSub?) then (D_TotalStreamInflowSub*I_RelFracObs) else 0) else 0 [2-D: subcatchment, observation point]

- D_RoutingTimeObs = if (I_RiverFlowTimeSub>0) then (I_RoutingDistanceObs / (I_RiverFlowTimeSub*I_RoutingVelocity_m_per_s * 3.6 * 24 * I_Tortuosity)) else 0 [2-D: subcatchment, observation point]

- D_SoilDischargePatch = min(max(0,D_SoilWaterPatch+D_InfiltrationPatch- D_ActETPatch- D_PercolationPatch),max(0,D_SoilWaterPatch – I_AvailWaterPatch)) [2-D: subcatchment, luc type]

- D_SurfaceFlowSub = if (L_LakeHereSub?) then (sum(I_DailyRainPatch[i,*])) else max(0,(sum(I_DailyRainPatch[i,*])- sum(D_InterceptPatch[i,*])- sum(D_InfiltrationPatch[i,*])- D_DeepInfiltrationSub)) [1-D: subcatchment]

- D_TotalStreamInflowSub = D_SurfaceFlowSub + (D_GWDischargeSub*(1- D_GWFracToLakeSub)+sum(D_SoilDischargePatch[i,*])) + (if (not I_DamHereSub?) then D_ReservoirOutflowSub  else 0) [1-D: subcatchment]

- G_DecayManureIncorportionPatch = min(G_SurfaceManurePatch+G_FaecesProductionPatch,G_SurfaceManurePatch* G_SurfaceManurePatchDecayFrac) [2-D: subcatchment, luc type]

- G_DecaySurfaceLitterIncorporationPatch = min(G_SurfaceLitterPatch+G_LitterDepositionPatch,G_SurfaceLitterPatch*G_SurfaceLitterDecayFrac) [2-D: subcatchment, luc type]

- G_FaecesProductionPatch = G_GrazingPatch*G_GrazingManureConverter [2-D: subcatchment, luc type]

- G_GrassCatch = sum(G_GrassBiomassPatch) [non-arrays]

- G_GrazingPatch = min(G_GrassBiomassPatch+G_GrowthRatePatch,if (G_GrassCatch>0) then (C_Stocking * C_DailyIntake * G_GrassBiomassPatch / G_GrassCatch) else 0) [2-D: subcatchment, luc type]

- G_GrowthRatePatch = G_WUE * I_GrassFracBiomassLUC * D_ActETPatch [2-D: subcatchment, luc type]

- G_LeafMortalityPatch = min(max(0,G_GrassBiomassPatch+G_GrowthRatePatch-G_GrazingPatch),(G_GrassBiomassPatch* G_GrassMortalityFrac) + (G_GrazingPatch* G_TramplingMultiplier)) [2-D: subcatchment, luc type]

- G_LitterDepositionPatch = G_LeafMortalityPatch* G_GrassLitterConverter [2-D: subcatchment, luc type]

- G_SurfaceCoverPatch = G_GrassBiomassPatch + G_SurfaceLitterPatch + G_SurfaceManurePatch [2-D: subcatchment, luc type]

- I_AvailWaterPatch = I_AvailWaterSub * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_ContributoryArea = if(I_RoutingDistanceObs > 0) then I_AreaSub else 0 [1-D: observation point]

- I_DailyRainPatch = I_DailyRainSub * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_InfiltrationDurationSub = min(24, I_RainDurationSub + D_InterceptDurationSub) [1-D: subcatchment]

- I_InterceptCapacityPatch = I_InterceptLUC * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_MaxInfiltrationPatch = I_MaxInfiltration * I_BD_BDRefSub  * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_MaxInfiltrationSubSoilPatch = I_MaxInfiltrationSubSoil * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_MeasuredDebit_mm_per_day = if(sum(I_ContributoryArea[*]) > 0) then ((I_MeasuredDebit_m3_per_sec * 24 * 3.6) / sum(I_ContributoryArea[*])) else 0 [1-D: observation point]

- I_PETPatch = I_ET_DailySet* I_RelETLUC_DailySet * I_RelAreaPatch [2-D: subcatchment, luc type]

- I_RainDurationSub = I_DailyRainSub/I_RainIntensitySub [1-D: subcatchment]

- I_RainIntensitySub = min(I_Rain_IntensityMax, max(I_Rain_IntensityMin, normal(I_Rain_IntensityMean, (I_Rain_IntensityMean * I_Rain_IntensityCV), I_Rain_GenSeed))) [1-D: subcatchment]

- I_RelFracObs = if (D_RoutingTimeObs <> 0) then (if (D_RoutingTimeObs > 0) then (min(1, (I_RiverFlowDispersalFactor / D_RoutingTimeObs))) else 0) else 1 [2-D: subcatchment, observation point]

- I_SoilSaturationPatch = (I_SoilFieldCapacitySub + I_AvailWaterSub) * I_RelAreaPatch [2-D: subcatchment, luc type]

- L_EvapLake = min(L_LakeVol,min(sum(L_LakeTranspirationSub*I_RelAreaSub),L_LakeVol)* I_LakeTranspirationMultiplier) [non-arrays]

- L_FloodingCurrent? = L_LakeLevel > L_FloodTresh [non-arrays]

- L_HEPP_kWh = 1000 * sum(I_AreaSub[*]) * L_HEPPWaterUseFlow / L_m3_per_kWh [non-arrays]

- L_HEPPOutflow = if (L_LakeLevel> L_LakeLevelFullHEPP) then L_HEPPDailyDemand else (if(L_LakeLevel> L_LakeLevelNoHEPP) then (L_HEPPDailyDemand * 0.5 * (1 + (max(0,(L_LakeLevel – L_LakeLevelHalfHEPP) )/(L_LakeLevelFullHEPP – L_LakeLevelHalfHEPP)))) else 0 ) [non-arrays]

- L_HEPPRelOperationalTime = if (I_Simulation_Time>0) then ((L_CumHEPPUse / L_HEPPDailyDemand) / I_Simulation_Time) else 0 [non-arrays]

- L_HEPPWaterUseFlow = min(max(0,L_LakeVol+L_InflowToLake-L_EvapLake-L_RiverOutflow),if (L_HEPP_Active?) then L_HEPPOutflow else 0) [non-arrays]

- L_InflowToLake = D_RiverFlowToLakeObs + D_GWToLakeObs [non-arrays]

- L_LakeLevel = if (sum(L_LakeAreaSub[*]) > 0) then ((L_LakeVol / (1000 * sum(L_LakeAreaSub[*]))) + L_LakeBottomElev) else 0 [non-arrays]

- L_LakeLevelExcess = L_LakeLevel  - (if (not L_HEPP_Active?) then L_LakeElevPreHEPP else 0) – (if (L_HEPP_Active?) then L_LakeElevPostHEPP else 0)  [non-arrays]

- L_LakeTranspirationSub = if (L_LakeHereSub?) then (max(0,D_ActETPatch[*,j]- I_PETPatch[*,j])), where j is water body else 0 [1-D: subcatchment]

- L_RiverOutflow = min(max(0,L_LakeVol+L_LakeOverflow-L_EvapLake),max(if(L_HEPP_Active?) then L_SanitaryFlow else 0,(L_LakeVol-(if (L_HEPP_Active?) then L_OutflowTresholdVolPostHEPP else 0)- (if (not L_HEPP_Active?) then L_OutflowTresholdVolPreHEPP else 0))* L_LakeOverFlowFrac* (1 + L_LakeLevelExcess ^ L_LakeOverFlow))) [non-arrays]

- O_BalanceCatch = O_CumInterceptCatch + O_CumETCatch + O_TotalStreamFlowCatch –O_DeltaGWCatch – O_DeltaSoilWaterStockCatch – O_CumRainCatch [non-arrays]

- O_CumSimulatedDebit = O_CumSimulatedDebit + O_SimulatedDebit_mm_per_day [1-D: observation point]

- O_DeltaGWCatch = O_CumGWCatch – sum(D_GWSub[*]*I_RelAreaSub[*]) [non-arrays]

- O_DeltaSoilWaterStockCatch = O_CumSoilWaterCatch – sum(sum(D_SoilWaterPatch[*,*]) * I_RelAreaSub) [non-arrays]

- O_FracBaseFlow = if (O_TotalStreamFlowCatch>0) then (O_CumGWDischargeCatch / O_TotalStreamFlowCatch) else 0 [non-arrays]

- O_FracSoilQuickFlow = if (O_TotalStreamFlowCatch>0) then (O_CumSoilDischargeCatch / O_TotalStreamFlowCatch) else 0 [non-arrays]

- O_FracSurfaceQuickFlow = if (O_TotalStreamFlowCatch>0) then (O_CumSurfaceFlowCatch / O_TotalStreamFlowCatch) else 0 [non-arrays]

- O_RelAvailWaterCatch = sum(sum D_RelAvailWaterPatch * I_RelAreaSub) [non-arrays]

- O_SimulatedDebit_mm_per_day = sum((D_DirectRiverFlowObs + D_DelayRiverFlowObs+D_RiverToLakeSameDayObs+D_RiverInflowToLakeObs) * I_RelAreaSub) [1-D: observation point]

- O_TotalStreamFlowCatch = O_CumGWDischargeCatch + O_CumSoilDischargeCatch + O_CumSurfaceFlowCatch [non-arrays]

- S_CompactionPatch = min(S_RelBDPatch+S_RippingSurfacePatch+S_StructureFormationPatch+S_SplashErosionPatch,if(S_MaxTrampling > 0)  then (1.3 – ((S_RelBDPatch * C_TramplingCompactionPatch) / S_MaxTrampling))) else 1.3 [2-D: subcatchment, luc type]

- S_RainAtSoilSurfacePatch = I_DailyRainPatch – D_InterceptPatch [2-D: subcatchment, luc type]

- S_SplashErosionPatch = if(I_RainDurationSub>0) then ((0*G_SurfaceCoverPatch* S_RainAtSoilSurfacePatch)/ I_RainDurationSub) else 0 [2-D: subcatchment, luc type]

- S_StructureFormationPatch = 0*S_RelBDPatch* G_SurfaceCoverPatch [2-D: subcatchment, luc type]

8.3.6. Stocks-parameters updated in the dynamic part

- C_Stocking = max(0, C_Stocking + C_StockingRate - C_Destocking) [non-arrays]

- D_AtmWaterPatch = max(0,D_AtmWaterPatch + D_ActETPatch + D_InterceptPatch) [2-D: subcatchment, luc type]

- O_CumDebitCatch = if I_ObserveAllSubcatchmentsObs? then O_SimulatedDebit_mm_per_day [non-arrays]

- D_CumRiverFlowObs = max(0, D_CumRiverFlowObs+ D_DelayRiverFlowObs+ D_DirectRiverFlowObs) [2-D: subcatchment, observation point]

- D_DelaySurfaceFlowToRiverObs_DailySet = update D_DelaySurfaceFlowToRiverObs_DailySet with new data on D_DelaySurfaceFlowObs by considering D_RoutingTimeObs and estimated day of flow to river

- D_GWSub = max(0,D_GWSub+sum(D_PercolationPatch[i,*]+ D_DeepInfiltrationSub- D_GWDischargeSub)) [1-D: subcatchment]

- D_ReservoirVolSub = max(0, D_ReservoirVolSub+ D_ReservoirInflowSub- D_ReservoirEvapSub- D_ReservoirOutflowSub) [1-D: subcatchment]

- D_SoilWaterPatch = max(0, D_SoilWaterPatch+ D_InfiltrationPatch- D_ActETPatch- D_PercolationPatch- D_SoilDischargePatch) [2-D: subcatchment, luc type]

- D_TotalDelayRiverFlowObs = max(0, D_TotalDelayRiverFlowObs+ D_DelaySurfaceFlowToRiverObs+ D_DirectSurfaceFlowObs- D_DelayRiverFlowObs- D_RiverInflowToLakeObs) [2-D: subcatchment, observation point]

- G_GrassBiomassPatch = max(0, G_GrassBiomassPatch+ G_GrowthRatePatch- G_GrazingPatch- G_LeafMortalityPatch) [2-D: subcatchment, luc type]

- G_SurfaceLitterPatch = max(0, G_SurfaceLitterPatch+ G_LitterDepositionPatch- G_DecaySurfaceLitterIncorporationPatch) [2-D: subcatchment, luc type]

- G_SurfaceManurePatch = max(0, G_SurfaceManurePatch+ G_FaecesProductionPatch- G_DecayManureIncorportionPatch) [2-D: subcatchment, luc type]

- L_CumEvapLake = max(0, L_CumEvapLake + L_EvapLake) [non-arrays]

- L_CumHEPPUse = max(0, L_CumHEPPUse + L_HEPPWaterUseFlow) [non-arrays]

- L_CumRiverOutflow = max(0, L_CumRiverOutflow + L_RiverOutflow) [non-arrays]

- L_LakeVol = max(0, L_LakeVol + L_InflowToLake - L_EvapLake - L_RiverOutflow - L_HEPPWaterUseFlow) [non-arrays]

- O_CumDeepInfiltrationCatch = max(0, O_CumDeepInfiltrationCatch + sum(D_DeepInfiltrationSub*I_RelAreaSub)) [non-arrays]

- O_CumETCatch = max(0, O_CumETCatch + sum(sum(D_ActETPatch)*I_RelAreaSub)) [non-arrays]

- O_CumGWCatch = max(0, O_CumGWCatch + (sum(D_GWSub * I_RelAreaSub) - O_CumGWCatch)) [non-arrays]

- O_CumGWDischargeCatch = max(0, O_CumGWDischargeCatch + sum(D_GWDischargeSub*I_RelAreaSub)) [non-arrays]

- O_CumHEPP_kWh = O_CumHEPP_kWh + L_HEPP_kWh [non-arrays]

- O_CumInfiltrationCatch = max(0, O_CumInfiltrationCatch + sum(sum(D_InfiltrationPatch)*I_RelAreaSub)) [non-arrays]

- O_CumInterceptCatch = max(0, O_CumInterceptCatch + (sum(sum(D_InterceptPatch) * I_RelAreaSub)) [non-arrays]

- O_CumPercolationCatch = max(0, O_CumPercolationCatch + sum(sum(D_PercolationPatch)*I_RelAreaSub)) [non-arrays]

- O_CumRainCatch = max(0, O_CumRainCatch + sum (I_DailyRainSub[*] * I_RelAreaSub[*])) [non-arrays]

- O_CumSoilDischargeCatch = max(0, O_CumSoilDischargeCatch + sum(sum(D_SoilDischargePatch)*I_RelAreaSub)) [non-arrays]

- O_CumSoilWaterCatch = max(0, O_CumSoilWaterCatch + (sum(sum(D_SoilWaterPatch) * I_RelAreaSub)) - O_CumSoilWaterCatch) [non-arrays]

- O_CumSurfaceFlowCatch = max(0, O_CumSurfaceFlowCatch + sum(D_SurfaceFlowSub*I_RelAreaSub)) [non-arrays]

- S_RelBDPatch = max(0, S_RelBDPatch+ S_StructureFormationPatch+ S_RippingSurfacePatch+ S_SplashErosionPatch- S_CompactionPatch) [2-D: subcatchment, luc type]

8.3.7. Non stocks-parameters updated in last the dynamic part

- I_Simulation_Time = I_Simulation_Time + 1 [non-arrays]

8.4. Related procedures in the script

- go

- initialize-before-simulation

- parameterize-daily-parameters

- parameterize-yearly-parameters

- report-and-plot

- simulate

- simulate-daily-dynamics

- simulate-grass-dynamics

- simulate-lake-dynamics

- simulate-patch-dynamics

- simulate-river-dynamics

- simulate-soil-structure-dynamics

- simulate-yearly-dynamics

- update-cattle-stocks

- update-data-set

- update-grass-stocks

- update-lake-stocks

- update-output-stocks

- update-patch-stocks

- update-river-stocks

- update-soil-structure-stocks

- update-stocks

- update-subcatchment-stocks

# [9] Reporting

9.1. Description

This is the last procedure, aiming at reporting simulation results, either in graphical plots or in output text files.

9.2. Parameters reported by this procedure

## In graphical plots:

- D_RiverFlowToLakeObs [1-D: observation point]

- I_MeasuredDebit_mm_per_day [non-arrays]

- L_HEPP_kWh [non-arrays]

- O_CumETCatch [non-arrays]

- O_CumGWDischargeCatch [non-arrays]

- O_CumRainCatch [non-arrays]

- O_CumSimulatedDebit [1-D: observation point]

- O_CumSoilDischargeCatch [non-arrays]

- O_CumSurfaceFlowCatch [non-arrays]

- O_SimulatedDebit_mm_per_day [1-D: observation point]

In output text file:

- D_GWDischargeSub [1-D: subcatchment]

- D_RiverFlowToLakeObs [1-D: observation point]

- D_SurfaceFlowSub [1-D: subcatchment]

- I_DailyRainSub [1-D: subcatchment]

- L_HEPP_kWh [non-arrays]

- O_BalanceCatch [non-arrays]

- O_SimulatedDebit_mm_per_day [1-D: observation point]

- sum(D_ActETPatch) [1-D: subcatchment]

- sum(D_SoilDischargePatch) [1-D: subcatchment]

9.3. Related procedures in the script

- ask-output-file-name

- initialize-before-simulation

- make-output-file-header

- parameterize-daily-parameters

- parameterize-yearly-parameters

- plot-HEPP

- plot-measured-river-flow-at-lake-inlet

- plot-measured-river-flow-at-observed-point

- plot-simulated-river-flow-at-lake-inlet

- plot-simulated-river-flow-at-observed-point

- plot-water-balance

- report-and-plot

- report-results

- setup

- simulate

- simulate-daily-dynamics

- simulate-grass-dynamics

- simulate-lake-dynamics

- simulate-patch-dynamics

- simulate-river-dynamics

- simulate-soil-structure-dynamics

- simulate-yearly-dynamics

- update-cattle-stocks

- update-data-set

- update-grass-stocks

- update-lake-stocks

- update-output-stocks

- update-patch-stocks

- update-river-stocks

- update-soil-structure-stocks

- update-stocks

- update-subcatchment-stocks

[E] List of parameters in GenRiver/NetLogo

| Parameters | Dimension | Type |
| --- | --- | --- |
| C_CattleSale | non-arrays | Converter |
| C_DailyIntake | non-arrays | Converter |
| C_DailyTramplingFrac | non-arrays | Converter |
| C_DeathRate | non-arrays | Converter |
| C_Destocking | non-arrays | Flow |
| C_NutritionMortalityThreshold | non-arrays | Converter |
| C_RelNutrition | non-arrays | Converter |
| C_Stocking | non-arrays | Stock |
| C_StockingRate | non-arrays | Flow |
| C_TramplingCompactionPatch | 2-D: subcatchment, luc type | Converter |
| D_ActETPatch | 2-D: subcatchment, luc type | Flow |
| D_AtmWaterPatch | 2-D: subcatchment, luc type | Stock |
| D_CumRiverFlowObs | 2-D: subcatchment, observation point | Stock |
| D_DeepInfiltrationSub | 1-D: subcatchment | Flow |
| D_DelayRiverFlowObs | 2-D: subcatchment, observation point | Flow |
| D_DelaySurfaceFlowObs | 2-D: subcatchment, observation point | Flow |
| D_DelaySurfaceFlowToRiverObs | 2-D: subcatchment, observation point | Flow |
| D_DelaySurfaceFlowToRiverObs_DailySet | 3-D: subcatchment, observation point, day |  |
| D_DirectRiverFlowObs | 2-D: subcatchment, observation point | Flow |
| D_DirectSurfaceFlowObs | 2-D: subcatchment, observation point | Flow |
| D_FeedingIntoLakeSub? | 1-D: subcatchment | Converter |
| D_GWDischargeSub | 1-D: subcatchment | Flow |
| D_GWFracToLakeSub | 1-D: subcatchment | Converter |
| D_GWLakeObs | 2-D: subcatchment, observation point | Converter |
| D_GWSub | 1D: subcatchment | Stock |
| D_GWToLakeObs | 1-D: observation point | Flow |
| D_GWUtilizedPatch? | 2-D: subcatchment, luc-type | Converter |
| D_GWUtilizationFractionPatch | 1D: subcatchment | Converter |
| D_InfiltrationPatch | 2-D: subcatchment, luc type | Flow |
| D_InterceptDripRate | non-arrays | Converter |
| D_InterceptDurationSub | 1-D: subcatchment | Converter |
| D_InterceptPatch | 2-D: subcatchment, luc type | Flow |
| D_MaxInterceptDripDuration | non-arrays | Converter |
| D_MaxReservoirVolSub | 1-D: subcatchment | Converter |
| D_PercolationPatch | 2-D: subcatchment, luc type | Flow |
| D_RelAvailWaterPatch | 2-D: subcatchment, luc type | Converter |
| D_ReservoirEvapSub | 1-D: subcatchment | Flow |
| D_ReservoirInflowSub | 1-D: subcatchment | Flow |
| D_ReservoirOutflowSub | 1-D: subcatchment | Flow |
| D_ReservoirUseFracSub | 1-D: subcatchment |  |
| D_ReservoirUseFracSub_Simulation | 3D: subcatchment, year, day |  |
| D_ReservoirUseFracSub_WarmUp | 3D: subcatchment, year, day |  |
| D_ReservoirVolSub | 1-D: subcatchment | Stock |
| D_RiverFlowToLakeObs | non-arrays | Flow |
| D_RiverInflowToLakeObs | 2-D: subcatchment, observation point | Flow |
| D_RiverToLakeSameDayObs | 2-D: subcatchment, observation point | Converter |
| D_RoutingTimeObs | 2-D: subcatchment, observation point | Converter |
| D_SoilDischargePatch | 2-D: subcatchment, luc type | Flow |
| D_SoilWaterPatch | 2-D: subcatchment, luc type | Stock |
| D_SurfaceFlowSub | 1-D: subcatchment | Flow |
| D_TotalDelayRiverFlowObs | 2-D: subcatchment, observation point | Stock |
| D_TotalDelayRiverFlowObs | 2-D: subcatchment, observation point | Stock |
| D_TotalStreamInflowSub | 1-D: subcatchment | Converter |
| daily-rain-data-set | 3-D: subcatchment, year, day |  |
| daily-rain-input-file-name-set | 1-D: subcatchment |  |
| daily-rain-year-set | 2-D: subcatchment, year |  |
| dynamic-subcathment-properties-data-set | 3-D: subcatchment, year, dynamic-subcathment-properties |  |
| dynamic-subcathment-properties-input-file-name-set | 1-D: subcatchment |  |
| dynamic-subcathment-properties-year-set | 2-D: subcatchment, year |  |
| G_DecayManureIncorportionPatch | 2-D: subcatchment, luc type | Flow |
| G_DecaySurfaceLitterIncorporationPatch | 2-D: subcatchment, luc type | Flow |
| G_FaecesProductionPatch | 2-D: subcatchment, luc type | Flow |
| G_GrassBiomassPatch | 2-D: subcatchment, luc type | Stock |
| G_GrassCatch | non-arrays | Converter |
| G_GrassLitterConverter | non-arrays | Converter |
| G_GrassMortalityFrac | non-arrays | Converter |
| G_GrazingManureConverter | non-arrays | Converter |
| G_GrazingPatch | 2-D: subcatchment, luc type | Flow |
| G_GrowthRatePatch | 2-D: subcatchment, luc type | Flow |
| G_LeafMortalityPatch | 2-D: subcatchment, luc type | Flow |
| G_LitterDepositionPatch | 2-D: subcatchment, luc type | Flow |
| G_SurfaceCoverPatch | 2-D: subcatchment, luc type | Converter |
| G_SurfaceLitterDecayFrac | non-arrays | Converter |
| G_SurfaceLitterPatch | 2-D: subcatchment, luc type | Stock |
| G_SurfaceManurePatch | 2-D: subcatchment, luc type | Stock |
| G_SurfaceManurePatchDecayFrac | non-arrays | Converter |
| G_TramplingMultiplier | non-arrays | Converter |
| generated-dynamic-subcathment-properties-data-set-for-simulation | 3D: subcatchment, year, dynamic-subcathment-properties |  |
| generated-dynamic-subcathment-properties-data-set-for-warm-up | 3D: subcatchment, year, dynamic-subcathment-properties |  |
| I_AreaSub | 1-D: subcatchment | Converter |
| I_AvailWaterPatch | 2-D: subcatchment, luc type | Converter |
| I_AvailWaterSub | 1-D: subcatchment |  |
| I_AvailWaterSub_Simulation | 2D: subcatchment, year |  |
| I_AvailWaterSub_WarmUp | 2D: subcatchment, year |  |
| I_BD_BDRefSub | 2-D: subcatchment, year |  |
| I_BD_BDRefSub_Simulation | 2D: subcatchment, year |  |
| I_BD_BDRefSub_WarmUp | 2D: subcatchment, year |  |
| I_ContributoryArea | 1-D: observation point | Converter |
| I_DailyRainPatch | 2-D: subcatchment, luc type | Converter |
| I_DailyRainSub | 1-D: subcatchment | Converter |
| I_DailyRainSub_Simulation | 3D: subcatchment, year, day |  |
| I_DailyRainSub_WarmUp | 3D: subcatchment, year, day |  |
| I_DamHereSub? | 1-D: subcatchment | Converter |
| I_ET | non-arrays | Converter |
| I_ET_DailySet | 1-D: day | Converter |
| I_GrassFracBiomassLUC | 1-D: luc type | Converter |
| I_GWRelFracSub | 1-D: subcatchment |  |
| I_GWRelFracSub_Simulation | 2D: subcatchment, year |  |
| I_GWRelFracSub_WarmUp | 2D: subcatchment, year |  |
| I_InfiltrationDurationSub | 1-D: subcatchment | Converter |
| I_InitRelGW | non-arrays | Converter |
| I_InitRelSoil | non-arrays | Converter |
| I_InterceptCapacityPatch | 2-D: subcatchment, luc type | Converter |
| I_InterceptionEffectOnTranspiration | non-arrays | Converter |
| I_InterceptLUC | 1-D: luc type | Converter |
| I_LakeTranspirationMultiplier | non-arrays | Converter |
| I_MaxDynGWSub | 1-D: subcatchment |  |
| I_MaxDynGWSub_Simulation | 2D: subcatchment, year |  |
| I_MaxDynGWSub_WarmUp | 2D: subcatchment, year |  |
| I_MaxInfiltration | non-arrays | Converter |
| I_MaxInfiltrationPatch | 2-D: subcatchment, luc type | Converter |
| I_MaxInfiltrationSubSoil | non-arrays | Converter |
| I_MaxInfiltrationSubSoilPatch | 2-D: subcatchment, luc type | Converter |
| I_MeasuredDebit_m3_per_sec | non-arrays | Converter |
| I_MeasuredDebit_mm_per_day | 1-D: observation point | Converter |
| I_ObserveAllSubcatchmentsObs? | 2-D: subcatchment, observation point | Converter |
| I_PercolationFraction | non-arrays | Converter |
| I_PETPatch | 2-D: subcatchment, luc type | Converter |
| I_PWPSub | 1-D: subcatchment |  |
| I_PWPSub_Simulation | 2D: subcatchment, year |  |
| I_PWPSub_WarmUp | 2D: subcatchment, year |  |
| I_Rain_GenSeed | non-arrays | Converter |
| I_Rain_IntensityCV | non-arrays | Converter |
| I_Rain_IntensityMax | non-arrays | Converter |
| I_Rain_IntensityMean | non-arrays | Converter |
| I_Rain_IntensityMin | non-arrays | Converter |
| I_RainDurationSub | 1-D: subcatchment | Converter |
| I_RainIntensitySub | 1-D: subcatchment | Converter |
| I_RainMultiplier | non-arrays | Converter |
| I_RelAreaPatch | 2-D: subcatchment, luc type |  |
| I_RelAreaPatch_Simulation | 3D: subcatchment, year, luc type |  |
| I_RelAreaPatch_WarmUp | 3D: subcatchment, year, luc type |  |
| I_RelAreaSub | 1-D: subcatchment | Converter |
| I_RelDroughtLUC | 1-D: luc type | Converter |
| I_RelETLUC | 1-D: luc type | Converter |
| I_RelETLUC_DailySet | 2-D: luc type, day | Converter |
| I_RelFracObs | 2-D: subcatchment, observation point | Converter |
| I_ReservoirDepth | non-arrays | Converter |
| I_RiverFlowDispersalFactor | non-arrays | Converter |
| I_RiverFlowTimeSub | 1-D: subcatchment |  |
| I_RiverFlowTimeSub_Simulation | 2D: subcatchment, year |  |
| I_RiverFlowTimeSub_WarmUp | 2D: subcatchment, year |  |
| I_RoutingDistanceObs | 2-D: subcatchment, observation point | Converter |
| I_RoutingVelocity_m_per_s | non-arrays | Converter |
| I_Simulation_Time | non-arrays | Converter |
| I_SoilFieldCapacitySub | 1-D: subcatchment |  |
| I_SoilFieldCapacitySub_Simulation | 2D: subcatchment, year |  |
| I_SoilFieldCapacitySub_WarmUp | 2D: subcatchment, year |  |
| I_SoilSaturationPatch | 2-D: subcatchment, luc type | Converter |
| I_Tortuosity | non-arrays | Converter |
| L_CumEvapLake | non-arrays | Converter |
| L_CumHEPPUse | non-arrays | Stock |
| L_CumRiverOutflow | non-arrays | Stock |
| L_EvapLake | non-arrays | Flow |
| L_FloodingCurrent? | non-arrays | Converter |
| L_FloodTresh | non-arrays | Converter |
| L_HEPP_Active? | non-arrays | Converter |
| L_HEPP_kWh | non-arrays | Converter |
| L_HEPPDailyDemand | non-arrays | Converter |
| L_HEPPOutflow | non-arrays | Converter |
| L_HEPPRelOperationalTime | non-arrays | Converter |
| L_HEPPWaterUseFlow | non-arrays | Flow |
| L_InflowToLake | non-arrays | Flow |
| L_LakeAreaSub | 1-D: subcatchment | Converter |
| L_LakeBottomElev | non-arrays | Converter |
| L_LakeElevPostHEPP | non-arrays | Converter |
| L_LakeElevPreHEPP | non-arrays | Converter |
| L_LakeHereSub? | 1-D: subcatchment | Converter |
| L_LakeLevel | non-arrays | Converter |
| L_LakeLevelExcess | non-arrays | Converter |
| L_LakeLevelFullHEPP | non-arrays | Converter |
| L_LakeLevelHalfHEPP | non-arrays | Converter |
| L_LakeLevelNoHEPP | non-arrays | Converter |
| L_LakeOverFlow | non-arrays | Converter |
| L_LakeOverFlowFrac | non-arrays | Converter |
| L_LakeTranspirationSub | 1-D: subcatchment | Converter |
| L_LakeVol | non-arrays | Stock |
| L_m3_per_kWh | non-arrays | Converter |
| L_OutflowTresholdVolPostHEPP | non-arrays | Converter |
| L_OutflowTresholdVolPreHEPP | non-arrays | Converter |
| L_QHEPP_m3_per_sec | non-arrays | Converter |
| L_QSanitaryFlow_m3_per_sec | non-arrays | Converter |
| L_RiverOutflow | non-arrays | Flow |
| L_SanitaryFlow | non-arrays | Converter |
| lake-parameters-data-set | 1-D: lake-parameters |  |
| lake-parameters-input-file-name | non-arrays |  |
| luc-fraction-data-set | 3D: subcatchment, year, luc type |  |
| luc-fraction-input-file-name-set | 1-D: subcatchment |  |
| luc-fraction-year-set | 2D: subcatchment, year |  |
| luc-number | non-arrays |  |
| max-daily-rain-data-set | 2-D: subcatchment, month |  |
| mean-monthly-rain-data-set | 2-D: subcatchment, month |  |
| min-daily-rain-data-set | 2-D: subcatchment, month |  |
| monthly-evapotranspiration-data-set | non-arrays |  |
| monthly-evapotranspiration-input-file-name | 1-D: month |  |
| monthly-relative-evapotranspiration-data-set | 2-D: month, luc type |  |
| monthly-relative-evapotranspiration-input-file-name | non-arrays |  |
| O_BalanceCatch | non-arrays | Converter |
| O_CumDebitCatch | non-arrays | Stock |
| O_CumDeepInfiltrationCatch | non-arrays | Stock |
| O_CumETCatch | non-arrays | Stock |
| O_CumGWCatch | non-arrays | Stock |
| O_CumGWDischargeCatch | non-arrays | Stock |
| O_CumHEPP_kWh | non-arrays |  |
| O_CumInfiltrationCatch | non-arrays | Stock |
| O_CumInterceptCatch | non-arrays | Stock |
| O_CumPercolationCatch | non-arrays | Stock |
| O_CumRainCatch | non-arrays (catchment level) | Stock |
| O_CumSimulatedDebit | 1-D: observation point | Converter |
| O_CumSoilDischargeCatch | non-arrays | Stock |
| O_CumSoilWaterCatch | non-arrays | Converter |
| O_CumSurfaceFlowCatch | non-arrays | Stock |
| O_DeltaGWCatch | non-arrays | Converter |
| O_DeltaSoilWaterStockCatch | non-arrays | Converter |
| O_FracBaseFlow | non-arrays | Converter |
| O_FracSoilQuickFlow | non-arrays | Converter |
| O_FracSurfaceQuickFlow | non-arrays | Converter |
| O_RelAvailWaterCatch | non-arrays | Converter |
| O_SimulatedDebit_mm_per_day | 1-D: observation point | Converter |
| O_TotalStreamFlowCatch | non-arrays | Converter |
| observation-point-number | non-arrays |  |
| observed-river-flow-data-set | 2-D: year, day |  |
| observed-river-flow-input-file-name | non-arrays |  |
| observed-river-flow-year-set | 1-D: day |  |
| p-monthly-wet-days-data-set | 2-D: subcatchment, month |  |
| rain-parameters-data-set | 1-D: rain-parameters |  |
| rain-parameters-input-file-name | non-arrays |  |
| reservoir-use-fraction-data-set | 3-D: subcatchment, year, day |  |
| reservoir-use-fraction-input-file-name-set | 1-D: subcatchment |  |
| reservoir-use-fraction-year-set | 2-D: subcatchment, year |  |
| river-parameters-data-set | 1-D: river-parameters |  |
| river-parameters-input-file-name | non-arrays |  |
| routing-distance-input-file-name | 1-D: subcatchment |  |
| S_CompactionPatch | 2-D: subcatchment, luc type | Flow |
| S_MaxTrampling | non-arrays | Converter |
| S_RainAtSoilSurfacePatch | 2-D: subcatchment, luc type | Converter |
| S_RelBDPatch | 2-D: subcatchment, luc type | Stock |
| S_RippingSurfacePatch | 2-D: subcatchment, luc type | Flow |
| S_SplashErosionPatch | 2-D: subcatchment, luc type | Flow |
| S_StructureFormationPatch | 2-D: subcatchment, luc type | Flow |
| scenario-file-name | non-arrays |  |
| sd-monthly-rain-data-set | 2-D: subcatchment, month |  |
| soil-parameters-data-set | 1-D: soil-parameters-data-set |  |
| soil-parameters-input-file-name | non-arrays |  |
| static-luc-properties-data-set | 1-D: static-luc-properties |  |
| static-luc-properties-input-file-name | non-arrays |  |
| static-subcathment-properties-data-set | 1-D: static-subcathment-properties |  |
| static-subcathment-properties-input-file-name | non-arrays |  |
| subcatchment-number | non-arrays |  |
| warm-up-duration | non-arrays |  |