SpatRain/NetLogo: How to use it?

[A] Basic need

- NetLogo 3.1.3 (recommended) and Java

[B] Preparing input data

- Prepare long term rainfall records in a tab delimited text file, where daily rainfall data in particular year from particular rainfall station are arranged in row-based order and other series from other years and other rainfall stations are arranged in column-based order.  Notice that the format only accepts 365-day data in each column, thus if the original data come from cardinal years, you must make the average of rainfall on February 28 and February 29. See the sample file included: “rain.tss” as the example.

- Prepare subcatchment boundary map in a xyz grid text file format, where x and y are the location in UTM format and z is the id of the subcatchment.  Please, code unconsidered grids as missing values (-9999). See the sample file included: “catch.xyz” as the example.

[C] Run the model

- Execute SpatRain.nlogo using NetLogo

- Set the storm properties parameters:

- storm-spread

- storm-edge

- storm-core-width

- storm-core-length

To produce patchy rain at subcatchment level without too many storm cores, use low storm-spread, high storm-edge, and medium storm-core-width and storm-core-length.  For example: using the above data sample (“rain.tss”), compatible patchy rainfall can be produced using storm-spread = 1, storm-edge = 60, storm-core-width = 7 and storm-core-length = 7. See the sample file included: “pat.out” as the example. Notice that if you decrease the storm-core-width and the storm-core-length, it will result too many storm cores and slow down the computation speed. Using the last setting, you can produce patchy rain at patch level, but since it has many storm cores, its patchiness will decrease at subcatchment level.

To produce homogenous rain at subcatchment level, simply increase the storm-spread and decrease the storm-edge. For example: using the above data sample (“rain.tss”), compatible patchy rainfall can be produced using storm-spread = 60, storm-edge = 1, storm-core-width = 7 and storm-core-length = 7. See the sample file included: “hom.out” as the example. Notice that most of homogenous rain only has single storm core, thus the computation speed will increase.

- Set data analyses parameters:

- class-number

- replicate

- sampled-patch-number

- do-spatial-analyses?

- sampling-distance

- sampling-day

To construct exceedance probability, you must define class-number.  To sample spatial distribution of simulated rain, you must define replicate. In the graphical plots (i.e. “Patch Rainfall” and “data Distribution”) the data are taken from some sampled points (red triangle in the view), thus you must define sampled-patch-number.  Further analyses are spatial analyses: semivariance analyses and scaling rule analyses. Those analyses will slow down the calculation speed.  Thus, if you just want to generate the rainfall without analysing its spatial pattern, you can swith-off do-spatial-analyses?.  If you do spatial analyses, you must define the sampling-distance and the sampling-day.

For example, class-number = 10, replicate = 10, and sampled-patch-number = 10.  If spatial analyses is carried out, use sampling-distance = 0.5 and sampling-day = 30 days.

- Set simulation parameters:

- simulation-duration

- report-for-GenRiver?

Define simulation-duration (years) to generate rainfall.  If you want to produce the results for GenRiver (subcatchment rainfall), you should switch-on report-for-GenRiver?.

- Set visualisation parameters:

- buffer

You can buffer your catchment boundary, so SpatRain can simulate larger area than your original boundary.  It is useful to solve the edge effect problem.

- Press the “setup” button.  First, you will be asked to select your rainfall data file (see B.1): "[SpatRain] Please, select file name of daily rainfall timeseries after this dialog! (*.tss)".  Just use the sample (“rain.tss”) for your first exercise. Then, you will be asked to select your subcatchment map file (see B.2): "[SpatRain] Please, select file name of subcatchment map in xyz format after this dialog! (*.xyz)".  Just use the sample (“catch.xyz”) for your first exercise. If you want to generate rainfall data for GenRiver need (by switching on report-for-GenRiver?), you will also be asked to define the output file name: "[SpatRain] Please, type the name of output file to save the simulation results at subcatchment level for GenRiver!" Just type the name, without extension, since SpatRain will automatically produce the extension for the file *.out.  For example: you can see the output samples: “pat.out” and “hom.out”.

- If you have no incompatible timeseries data with your assumed storm properties in all months, the setup will be ended by this message, you can proceed to the main simulation by pressing “go” button. But, if you have incompatible timeseries data with your assumed storm properties in one of the months, you have to adjust your storm properties (see C.2).

- Wait the simulation until you got this message: "[SpatRain] Finished. You must setup the model again to run other simulations."  During the simulation, you can see some graphical plots.  To save the data from graphical plots into csv file, right click the plot and select “Export...”.

- If you report the results for GenRiver, you can open the output file using MS Excel (choose tab delimited text file).  Your output files should have extension *.out and are stored in your current working directory. For example: you can see the output samples: “pat.out” and “hom.out”.  Currently, SpatRain does not generate rainfall data specifically for dry, normal and wet years.  Thus, should you want to use the generated data with specific yearly patterns, you must select manually from the simulated dataset.