GenRiver/NetLogo: How to use it?

[A] Basic need

- NetLogo 3.1.3 (recommended) and Java

- MS Excel

[B] Run the model

- Execute Genriver.nlogo using NetLogo

- Set the running control parameters:

- Whether you want to warm up the model before simulate it.

- Wheter you want to report simulation results in external text file.

- Simulation starting year (yyyy).

- Simulation duration (years).

- Warming up duration (years).

- Id of one observation point where you want to see the simulation result on total river flow (mm) into graphical plot (“Observed point”) or into a view and lake inflow into graphical plot (“Lake inlet”). You should refer the id of observation point from “Page2” of “GenRiver_NetLogo.xls”.

- Id of water body in land use/cover classes to identify lake existance. You should refer the id of observation point from “Page3” of “GenRiver_NetLogo.xls”.

- Whether you want to plot measured debit in “Observed point”.

- Whether you want to plot measured debit in “Lake inlet”.

- Whether you want to visualise the dynamic of water level at the observed-point in the view.  You should define its scale to fit into the view window size.

- Whether you want to simulate HEPP.

- Press the “setup” button.

- You will get this message: “[GenRiver] Please, select the name of scenario file after this dialog!”. Select a scenario file (developed in “Page20” of “GenRiver_NetLogo.xls”) from your working directory.  “GenRiver_NetLogo.xls” automatically give the file name with the extension: *.par. Just use the sample provided: scen.par for exercise.

- Then, you will get this message: “"[GenRiver] Please, define the name of output file to save the simulation results!".  Put any name without extension.  The program will automatically save your output files with *.out extension.

- The program will read the input data and validate the format.  You will have some error messages or running error if your format is wrong.  Returns to “GenRiver_NetLogo.xls” to prepare the data properly.

- The setup process may take time, depending the size of your data and the warming up duration.  Just wait patiently until you got this message: "[GenRiver] The model is now ready for simulation. Press go button to proceed the simulation."

- Now, press the “go” button.

- When simulation is done, you will get this message: "[GenRiver] Simulation is done."

- If you stored the simulation result, you can open it using MS Excel.  Select the file name from your working directory.  The file has the extension *.out.  Use tab delimiter.  For the next run, you can store the results in the same file name.  Do not worry! It will keep the previous results.

- If you want to repeat simulation, you should start from step 2.

[B] Preparing input data

- Execute “Genriver_NetLogo.xls” using MS Excel.

- “Page1”:

- Define number of subcatchments

- Define number of land use/cover classes

- Define number of observation points

- Define number of years in time series of measured debit.

- “Page2”:

- Subcatchment id is automatically generated by the macro.  You cannot modify this id, since it will be used in the NetLogo program to identify sequence number of an item in a list or multiple lists.

- Define the name of subcatchments to ease you to parameterise in the next pages.

- Define number of years in rainfall time series (“Page5”).

- Define number of years in luc fraction timeseries (“Page6”).

- Define number of years in soil properties timeseries (“Page7”).

- “Page3”:

- Land use/cover id is automatically generated by the macro.  You cannot modify this id, since it will be used in the NetLogo program to identify sequence number of an item in a list or multiple lists.

- Define the name of land use/cover type to ease you to parameterise in the next pages.

- “Page4”:

- Observation point id is automatically generated by the macro.  You cannot modify this id, since it will be used in the NetLogo program to identify sequence number of an item in a list or multiple lists.

- Define the name of observation points to ease you to parameterise in the next pages.

- “Page5”:

- Input data on daily rainfall timeseries on each subcatchment.  You must provide the year (yyyy) in the first row.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.  This timeseries will be stored in multiple files, separated by subcatchment.  Suffix of the resulted file names indicates the subcatchment id. Example: you give a name “rain” and you have 3 subcatchment.  The resulted file names are: “rain 1.dat”, “rain 2.dat” and “rain 3.dat”.

- You can also use the output from SpatRain for this data requirement (in preparation).

- “Page6”:

- Input data on yearly land use/cover fraction timeseries on each subcatchment.  You must provide the year (yyyy) in the first row.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.  This timeseries will be stored in multiple files, separated by subcatchment.  Suffix of the resulted file names indicates the subcatchment id. Example: you give a name “luc” and you have 3 subcatchment.  The resulted file names are: “luc 1.dat”, “luc 2.dat” and “luc 3.dat”.

- You can also use the output from the FALLOW model for this data requirement (in preparation).

- “Page7”:

- Input data on yearly dynamic soil properties timeseries on each subcatchment.  You must provide the year (yyyy) in the first row.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.  This timeseries will be stored in multiple files, separated by subcatchment.  Suffix of the resulted file names indicates the subcatchment id. Example: you give a name “dyn” and you have 3 subcatchment.  The resulted file names are: “dyn 1.dat”, “dyn 2.dat” and “dyn 3.dat”.

- “Page8”:

- Input data on daily measured reservoir use fraction timeseries on each subcatchment.  You must provide the year (yyyy) in the first row.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.  This timeseries will be stored in multiple files, separated by subcatchment.  Suffix of the resulted file names indicates the subcatchment id. Example: you give a name “res” and you have 3 subcatchment.  The resulted file names are: “res 1.dat”, “res 2.dat” and “res 3.dat”.

- “Page9”:

- Input data on daily measured river debit  timeseries on each subcatchment.  You must provide the year (yyyy) in the first row.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.  This timeseries will be stored in multiple files, separated by subcatchment.  Suffix of the resulted file names indicates the subcatchment id. Example: you give a name “deb” and you have 3 subcatchment.  The resulted file names are: “deb 1.dat”, “deb 2.dat” and “deb 3.dat”.

- “Page10”:

- Input data on static subcatchment properties.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page11”:

- Input data on routing distance of each subcatchment from each observation point.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page12”:

- Input data on monthly potential evapotranspiration.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page13”:

- Input data on relative monthly potential evapotranspiration by land use/cover type.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page14”:

- Input data on static land use/cover properties.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page15”:

- Input data on lake parameters.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page16”:

- Input data on rain parameters.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page17”:

- Input data on river parameters.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page18”:

- Input data on soil parameters.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page19”:

- Input data on cattle and grazing parameters.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.dat.

- “Page20”:

- Input file names of input data required by the model for one scenario.

- If you already have some *.dat files with different setting, you can directly go to “Page20” to construct some scenarios.

- To save the data, provide a file name, without extension.  The data will automatically has extension *.par.  The model (GenRiver.nlogo) will ask you this metadata file.